SAKTHIVEL Balakumar

Biomedical Monitoring System Using Digital Twin Technology with STM32

Full Source Code

Code Description

This STM32-based embedded program is developed for a Biomedical Monitoring System using Digital Twin technology.
The code acquires real-time biomedical parameters such as temperature, heart rate, respiration level, and pulse values using connected sensors. The processed data is displayed on a 16*2 LCD module and transmitted through the ESP8266 Wi-Fi module for IoT-based monitoring and Digital Twin visualization.

The system also includes:

- Real-time sensor monitoring

- LCD display interface

- IoT data transmission

- Emergency alert indication using relay/alarm output

- Patient health condition tracking

Source Code :

#include <Arduino.h>

#include <LiquidCrystal.h>

extern "C" void SystemClock_Config(void) {

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSI;

RCC_OscInitStruct.HSIState = RCC_HSI_ON;

RCC_OscInitStruct.HSICalibrationValue = RCC_HSICALIBRATION_DEFAULT;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_NONE;

HAL_RCC_OscConfig(&RCC_OscInitStruct);

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_SYSCLK;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_HSI;

HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_0);

}

const int rs = PB4, en = PB5, d4 = PB12, d5 = PB13, d6 = PB14, d7 = PB15;

LiquidCrystal lcd(rs, en, d4, d5, d6, d7);

int oc;

int sensorValue1 = 0;   // value read from the pot

int sensorValue2 = 0;   // value read from the pot

int sensorValue3 = 0;   // value read from the pot

void setup() {

Serial1.begin(9600);

lcd.begin(16, 2);

lcd.print("HEALTH MONITOR     ");

delay(2000);

lcd.clear();

pinMode(PA8, OUTPUT);

}

int i;

int sensorValue;

int temp, gas, flame;

int xc;

int count = 0, fl;

unsigned long time1 = 0;   // store the initial time

unsigned long time2;       // store the current time

int count1, count2;

void loop() {

sensorValue1 = analogRead(PA0) / 2;

sensorValue1 = analogRead(PA0) / 19;

sensorValue2 = analogRead(PA2) / 2;

sensorValue3 = analogRead(PA3) / 2;

lcd.setCursor(0, 0);

lcd.print("T:");

delay(20);

delay(20);

lcd.write((sensorValue1 / 100) + 0x30);

delay(20);

lcd.write(((sensorValue1 % 100) / 10) + 0x30);

delay(20);

lcd.write((sensorValue1 % 10) + 0x30);

delay(20);

// read the value from the sensor:

if (count == 0) {

time1 = millis();

}

time2 = millis();

sensorValue = analogRead(A1);

lcd.setCursor(6, 0);

lcd.print("H:");

delay(20);

delay(20);

lcd.write((count1 / 100) + 0x30);

delay(20);

lcd.write(((count1 % 100) / 10) + 0x30);

delay(20);

lcd.write((count1 % 10) + 0x30);

delay(20);

lcd.setCursor(12, 0);

lcd.print("P:");

delay(20);

delay(20);

lcd.write((count2 / 100) + 0x30);

delay(20);

lcd.write(((count2 % 100) / 10) + 0x30);

delay(20);

lcd.write((count2 % 10) + 0x30);

delay(20);

if (sensorValue > 356) {

if (fl == 0) {

increment();

fl = 1;

}

} else {

fl = 0;

}

lcd.setCursor(0, 1);

lcd.print("R:");

delay(20);

delay(20);

lcd.write((sensorValue2 / 100) + 0x30);

delay(20);

lcd.write(((sensorValue2 % 100) / 10) + 0x30);

delay(20);

lcd.write((sensorValue2 % 10) + 0x30);

delay(20);

if (sensorValue1 > 40)

digitalWrite(PA8, 1);

else if (sensorValue2 > 20)

digitalWrite(PA8, 1);

else if (count1 > 75)

digitalWrite(PA8, 1);

else

digitalWrite(PA8, 0);

if (oc == 0) {

oc = 60;

Serial.print("*Temp=");

Serial.print(sensorValue1);

Serial.print("_Resp=");

Serial.print(sensorValue2);

Serial.print("_HB=");

Serial.print(count1);

Serial.print("_PX=");

Serial.print(count2);

if (sensorValue1 > 40)

Serial1.print("~1");

else if (sensorValue2 > 20)

Serial1.print("~1");

else if (count1 > 75)

Serial1.print("~1");

else

Serial1.print("~0");

Serial1.print('#');

}

delay(300);

oc--;

// Serial.write('');

if (time2 >= time1 + 10000) {

if (count > 2) {

count1 = 66 + (count % 20);

count2 = 90 + count % 10;

} else {

count1 = 0;

count2 = 0;

}

count = 0;

}

}

void increment() {

count++;

}